| NEW 02 | 护栏式全不锈钢旋转升降拖猪车 Guardrail Stainless-Steel Rotating & Lifting Pig Trailer |
| --- | --- |

| 产品介绍 PRODUCT OVERVIEW 在全不锈钢旋转升降底盘基础上增加双侧护栏和宽幅承载滑板，为装载、提升及移动过程提供更清晰的导向与侧向防护。大直径驱动轮配合前端万向轮，兼顾承载稳定性与转向灵活性，适用于规模化猪场的日常转运及清洁消毒环境。 Built on an all-stainless-steel rotating and lifting chassis, this version adds full-length side guardrails and a wide loading deck for improved guidance and lateral containment during loading, lifting and transport. Large drive wheels and a front swivel caster balance stability with maneuverability, making the unit suitable for routine transfer work on commercial pig farms. 整车材质 Material; 全304不锈钢结构 All-304 stainless-steel construction. 外形尺寸 Dimensions; 长90cm、宽60cm、高度倾斜可调95cm-135cm Length: 90 cm, Width: 60 cm, Inclined height adjustable from 95 cm to 135 cm. 结构特点 Structure; 宽幅滑板、双侧护栏、旋转升降底盘 Wide deck, dual side guardrails and rotating lifting chassis. 适用场景 Applications; 规模猪场、栏位转运、病死猪无害化转运 Commercial pig farms, pen-side transfer and mortality handling. | 拖猪方式 Loading directions; 左侧、右侧、正前方均可对接 Can be approached from the left, right, or straight ahead.. 产品特点 Key functions; 电动行走、电动提升、电动仰俯、轻便旋转 Powered travel, powered lifting, powered tilt, and easy manual rotation.. 电池箱规格 Battery box; 201不锈钢电池箱，约33.5×19×24cm，底板厚1mm 201 stainless-steel battery box, approx. 33.5 × 19 × 24 cm, with a 1 mm base plate.. 钢丝绳规格 Wire rope; 304不锈钢，长10m，直径6mm 304 stainless-steel wire rope, 10 m long, Ø6 mm.. 前轮规格 Front wheels; 400-6充气轮，直径约330mm 400-6 pneumatic wheels, approx. Ø330 mm.. 后轮规格 Rear wheel; 350-4实心万向轮，直径约250mm 350-4 solid swivel caster, approx. Ø250 mm.. 电池规格 Battery; 4块12V 20Ah全新铅酸蓄电池 Four new 12 V 20 Ah lead-acid batteries.. 充电器 Charger; 48V 20A充电器 48 V 20 A charger.. 持续工作时间 Operating time; 持续使用6小时以上 More than 6 hours of continuous operation.. 行走系统 Travel system; 最大功率1000W，防水等级IP54 Maximum power: 1000 W; ingress protection: IP54.. 提升系统 Lifting system; 功率1000W，垂直拉力800kg，防水等级IP67 Power: 1000 W; vertical pulling force: 800 kgf; ingress protection: IP67.. 仰俯系统 Tilting system; 48V电动推杆，最大推力700kg，防水等级IP56 48 V linear actuator; maximum thrust: 700 kgf; ingress protection: IP56.. 控制系统 Control system; 急停、电源、提升、仰俯开关及电量显示表 Emergency stop, master power, lift and tilt switches, plus a battery level indicator.. 操控方式 Driving controls; 左侧刹车把手控制旋转定位销；右侧转把控制行走 The left brake lever operates the rotary locking pin; the right twist throttle controls travel.. 行走速度 Travel speed; 前进最高5km/h，倒车最高3km/h Forward speed up to 5 km/h; reverse speed up to 3 km/h.. 整车防护 Overall protection; 整车IP54，可防雨淋和喷洒消毒液，禁止浸泡 Overall IP54 protection. Suitable for rain exposure and disinfectant spraying; do not immerse.. |
| --- | --- |